# 1. Product Identification and Company Information

# Product Name:			Piezo Pink Cleaning Solution (PN 01-0003-009)

# Chemical family/Product type:	Ink Jet Fluid; water based; non-flammable

# Manufacturer:			Spectracorp

# Address:				118 South Pecos Street

# Denver, Colorado, 80223

# Emergency Phone Number:	303-371-6979

# Fax Number:				303-371-6688

# CAS Number:			Mixture/ Trade Secret

# Last update:				12/21/2014

# This SDS has been prepared for the purposes of Hazard Communication, under 29 CFR 1910.1200

# 2. Hazard Identification

# EMERGENCY OVERVIEW: WARNING! May cause irritation to skin, eyes and respiratory tract.

# Target Organs:			Skin

# GHS Classification:		Skin irritation; Eye irritation

# GHS Label Elements, including Precautionary Statements

# Pictograms:

# Signal Word:			Warning

# Hazard Statements:		Causes skin irritation.

# Causes eye irritation.

# Precautionary Statements:	If In Eyes: Rinse cautiously with water  for several minutes. Remove contact lenses. Continue rinsing.

# Potential Health Effects:

# Inhalation: May cause respiratory tract irritation.

# Skin: May cause allergic skin reaction.

# Eyes: May cause slight irritation.

# Ingestion: May be harmful if swallowed.

# 3. Composition and Information on Ingredients

| # Identification | # CAS Number | # Weight % |  |
| --- | --- | --- | --- |
| # Water # Propylene glycol # Polyethylene glycol # Proprietary Antimicrobial/Antifungal agent | # 7732-18-5 # 57-55-6 # Not Established # Not Established | # < 75 # < 20 # < 15 # < 1 |  |

# 4. First Aid

# Skin:				Wash with soap and water and rinse thoroughly.

# Eyes:                                   Flush eyes and under eyelids for 15 minutes with water.

# Seek medical attention if irritation or redness persists.

# Inhalation:	Move to fresh air. Seek medical attention if irritation persists.

# Ingestion:			Give water to dilute and get medical attention.

# 5. Fire Fighting Measures

# NFPA Rating: 	Health – 1, Flammability – 0, Reactivity – 0, Special Hazards - 0

# Extinguishing Media:	Water Spray, carbon dioxide, dry chemical.

# Hazardous Combustion Products:   	Material may burn in fire conditions releasing Products which may be toxic (SO2 CO,CO2, and volatile organics).

# Fire Fighting Procedures:                   Wear approved MSHA/NIOSH breathing apparatus and full protective clothing.

# 6. Accidental Release

# Spill Response:	Collect spilled material and wash area with water.

# Neutralizing Chemicals:		      None Required

# Waste Disposal:			      Follow all federal, state and local Regulations

# 7. Handling and Storage

# Handling: Avoid any contact with skin, eyes and clothing, do not breath vaporized fluid, wash hands and any areas that come in contact with fluid.

# Storage: Do not store in extreme heat (100 degrees or hotter) or extreme cold (40 degrees or colder).  Do not store in direct sunlight. Keep out of reach of children.

# 8.Exposure Controls/Personal Protection

# In accordance with good industrial practices, handle with care and avoid personal contact.

# Wear protective gloves, safety goggles, and other protective clothing.

# Avoid contact with skin, eyes, and clothing.

# Mechanical exhaust required.

# Slight Irritant.

# Keep tightly closed.

# Keep from freezing.

| # Identification | # CAS Number | # OSHA NIOSH ACGIH |  |
| --- | --- | --- | --- |
| # Water # Propylene glycol # Polyethylene glycol # Proprietary Antimicrobial/ # Antifungal agent | # 7732-18-5 # 57-55-6 # Not Established # Not Established | # None None None # 10 mg/m3 TWA Not Established 10 mg/m3 TWA # Not Established Not Established Not Established # Not Established Not Established Not Established |  |

# Eye Protection: Use chemical safety glasses or goggles and/or a full face shield where splashing is possible. Maintain eye wash fountain and quick-drench facilities in work area

# Respiratory Protection: No special respiratory protection required if adequate ventilation is provided.

# Ventilation: Provide sufficient mechanical ventilation.

# Protective Clothing: Wear chemical-resistant gloves and protective clothing appropriate for the risk of exposure. Avoid unnecessary skin contact. Wash hands thoroughly after handling.

# Work and Hygienic Practices: Practice good hygiene and maintain a clean work environment.

# 9. Physical Data

# Appearance						Clear Liquid

# Solubility in Water					Miscible

# Freezing Point					Less than 0 degrees Celsius

# Boiling Point						Greater than 100 degrees Celsius

# PH (100%)						6.4 – 10.0

# Specific Gravity, 25 C				1.01 - 1.10

# Viscosity, cps @ 25 C	                                 2.0 - 6.0

# Flash Point, C					Not Established

# Volatile Organic Compounds (VOC)		Not Established

# Percent Volatiles					Not Established

# 10.Reactive Data

# Reactivity:			No dangerous reaction under normal conditions known to occur.

# Chemical Stability:		Stable under normal conditions.  Will not polymerize.

# Incompatibility:		Strong Oxidizing agents, Strong Acids

# Hazardous Decomposition Products:	Carbon monoxide, Carbon Dioxide, Sulfur Oxides.

# 11. Toxicological Information

# The components of this product are not listed by the NTP, IARC, or regulated as a carcinogen of OSHA.  The product has not been tested for ingestion but will likely cause irritation if ingested based upon the ingredients contained in the formula.  Individual compounds are listed below in pure form.  This is a mixture and concentration levels are dramatically reduced for the fluid over the pure chemical toxicological values.

| # Identification | # CAS Number | # Acute Toxicity # Ingestion LD50 Rat Inhalation |  |
| --- | --- | --- | --- |
| # Water # Propylene glycol # Polyethylene glycol # Proprietary Antimicrobial/ # Antifungal agent | # 7732-18-5 # 57-55-6 # Not Established # Not Established | # No information available No information available # > 20,000 mg/kg > 20,000 mg/kg # Not Established Not Established # Not Established Not Established |  |

# 12. Ecological Information

# Environmental Hazard.  There are no known environmental hazards contained in the product.  The solution should not be disposed of except in small quantities in sanitary sewer systems.

# 13. Disposal Considerations

# Waste Disposal: Dispose of waste in accordance to all local, state, provincial, and national requirements.

# 14. Transport Information

# DOT:	Proper Shipping Name: Ink Jet Fluid/Ink

# Non-Hazardous for Transport: This substance is considered to be nonhazardous

# For transport and not regulated under DOT, IATA, ADR, or RID.

# 15. Regulatory Information

# CALIFORNIA PROPOSITION 65: This product contains no chemicals know to the state California to cause cancer, birth defects, or other reproductive harm.

# SARA Title III 311/312: This product has been reviewed according to the EPA Hazard Categories” promulgated under section 311 and 312 of SARA TITLE III, and is considered, under applicable definition, to be an immediate/acute health hazard.

# EEC SYMBOLS AND INDICATIONS OF DANGER: None

# R-PHRASES: R36/38 – Irritating to eyes and skin.

# S-PHRASES: S24/25 – Avoid contact with skin and eyes.

# WHMIS HAZARD SYMBOLS: None

# TOXIC Substance Control Act Status

# All ingredients are listed on the Environmental Protection Agency’s Toxic

# Substance Control Act (TSCA ) Inventory.

# 16. Other

# The above information is believed to be correct but does not purport to be all-inclusive and shall only be used as a guide. Spectracorp offers this information as a service to our customers and shall not be held liable for any damage resulting from handling or from contact with the above product.

# END OF SDS